Goal: MicroSD Card module with RP2350

Components:

- RP2350

  - Two hardware SPI controllers

- MicroSD Socket: Molex 104031-0811

  - Has active microSD connector, surface-mount, card-detection switch, easily available

- 3.3V Regulator: AP2112K-3.3TRG1

- Capacitors

  - 10uF electrolytic for input and output

  - 0.1 uF ceramic for SD decoupling

  - 4.7uF for SD bulk

- Resistors

  - Pull up resistors

  - Series for reducing ringing on SPI signals

- ESD Protection

  - TPD4E05U06

- Header pins

- Through-hole pads

Design:

https://www.nxp.com/docs/en/application-note/DRM125.pdf

Overall design is a 5V input with 3V regulator

Pin Setups:

- RP2350

  - GP18 to microSD CLK

  - GP19 to microSD CMD

  - GP16 to DAT0

  - GP17 to CD/DAT3

  - GP20 for manual card detection

- Molex Socket

  - Pin 1 – Floating

  - Pin 2 – CS

  - Pin 3 – MOSI

  - Pin 4 – 3.3V

  - Pin 5 – SCK

  - Pin 6 – GND

  - Pin 7 – MISO

  - Pin 8 – Floating

  - SW1/2 – GPIO input and GND

  - Shell tabs – GND

- Voltage regulator

  - Utilize capacitors for input fluctutaions and filter noise